Wenzhou Safewire Electric CO.,Ltd.

SPECIFICATION  SHEET

PRODUCT NAME:    Automatically pop up powerbar

PRODUCT No.:       FZ-527B

Address:2nd floor, Wealth Garden, Liushi town, Yueqing,  Wenzhou,China ,325604

Tel:0086-577-61725369

Mob:0086-18612380398

ZIP:325604

（E-mail）：info@safewirele.com

(WEB):  http://www.safewirele.com

Environmental requirements

| Serial No. | Parameter | Description |
| --- | --- | --- |
| 1 | Operating temperature | -10℃~+40℃;Operating |
| 2 | humidity | 20%~90% RH |
| 3 | Storage temperature | -20℃~+85℃;storage |
| 4 | humidity | 10%~95% RH |
| 5 | Resistance to pressure | ≥3Kv/AC 1min |
| - 6 | Isolation resistance | 500v DC/100MΩ |

- Technical parameters :

| Serial No. | Parameter | Description |
| --- | --- | --- |
| 1 | Material | Aluminum 6063, PC zinc alloy |
| 2 | Optional color | black |
| 3 | Panel Dimension(mm) | Ø120 |
| 4 | Cutout(mm) | Ø100 |
| 5 | Height when closed(mm) | 435 |
| 6 | Height when pop up(mm) | 667 |
| 7 | Capacity of modules /Interface size | 8 ways 45*45mm Legrand’s Mosaic modules |
| 8 | Configuration | Power outlet+Data outlet or USB charging : Customized as required |
| 9 | Cable length | 1.5m or required length |
| 10 | Time of engine motor | MAX 1000h |

Application：

- Office and education

- Industry and Engineering

- Presentation and retail;

- Orientation and safety;

- Transit areas and parking;

- Hotels, hospitality and wellness;

- Health, sport and leisure;

- Art and culture;

- Product Show:

- Features:

- Automatically on/off,Optional wireless charger of QI

- 2.1A/4.2A fast USB charger and blue tooth speaker outlet etc

- International 45mm Legrand’S modules/outlets

- Aluminum alloy 6063 reliable light housing

- optional 1.5 M International IEC plug and cora of 1.0 mm²square or 1.5mm² square.

- LED control panel design.

FZ-527B Outline drawing:

Package:4 Pcs inner box into a master carton.Poly bag with box packing

.

Mounting ring thread

（1）

（2）

（3）

Installation Process

- Open a circular hole with a diameter of 100mm.

- After removing the buckle，insert the product into the hole in the tabletop.adjust the direction and fix the product on the buckle at the bottom of the tabletop.

- Connection cable and tighten the screw ring to use

Available for F type 45*45mm type or D type22.5*45mm international sockets or modules types:

Attentions:

- 1.Prevent from corrosive liquid, gas (strong acid, strong alkali, paint)and humidity.

- 2. All dimension quoted are nominal only. Drawings , images and text are for illustrative perposes only and product details and dimension are subject to change without notice.

Safety Warning:

- For the protection and safety of you and others,please read this manual before installation.If you have something you don't understand when you install it,please contact us in any time.

- Please noted that the sum of current (power) should not exceed the rated current (power) of the product.3*1.0 square power cable with a maximum power of 2000W,3*1.5 square power line maximum power 3000W

- Please install this product in a location that is out of reach of children to ensure personal safety.

- To avoid electric shock, fire, and electrical failure ,please don't install this product in a humid environment.

- To avoid electric shock and fire,please don't damage the power cord and power outlet.

- To avoid electric shock and the expiration of the warranty period,please don't disassemble this product.

- To avoid electric shock and fire caused by electrical leakage,connect a cable with a grounding.